# Finglas Village Improvement Scheme.

# Written Description of Drawings.

# Date of Issue: 24 April 2026

Table of Contents.

1.	Overview.	3

2.	Drawing number one shows the North Road to Five arm junction along Seamus Ennis Road.	3

3.	Drawing number two shows the junction of McKee Avenue, Jamestown Road (south and north) and Seamus Ennis Road, commonly known as the five -arm junction to Saint Canice’s Girls National School.	4

4.	Drawing number three shows Seamus Ennis Road between Saint Canice’s National School and the junction with Clune Road.	6

5.	Drawing number four shows the junction of Seamus Ennis Road, Clune Road and Glasanon Road.	7

6.	Drawing number five shows Jamestown Road (South) and the junction of Main Street.	8

## Overview.

The Finglas Village Improvement Scheme forms part of a wider active travel route: The Finglas to Killester Active Travel Scheme. The project includes changes to the walking, wheeling and cycling infrastructure along the route. The scheme design drawings are described in the following document.

All references to “The Project” or “The Scheme” in this document refers to the Finglas Village Improvement Scheme only.

The scheme covers an area between the junction of Seamus Ennis Road/North Road (to the west) and the junction of Seamus Ennis Road / Clune Road / Glasanaon Road (to the east). It also includes Jamestown Road between the junction with Main Street (to the south) and the junction with Seamus Ennis Road (to the north). The scheme length is 650 metres.

The drawing pack includes five drawings of the Finglas Village Improvement Scheme which set out the new walking and cycling infrastructure along the route. The cycle tracks will be surfaced with red asphalt.

## Drawing number one shows the North Road to Five arm junction along Seamus Ennis Road.

Drawing number one illustrates the western side of the scheme along Seamus Ennis Road, between its junctions with North Road and Jamestown Road and Mckee Avenue, commonly referred to as the five-arm junction. On the northern side of Seamus Ennis Road are Alpha Care Hardware, the Shamrock Lodge Bar, and SuperValu. On the southern side are the Drogheda Mall car park, the Drogheda Mall retail units, and the Bank of Ireland.

Travelling east from the North Road junction towards Jamestown Road, it is proposed to provide a one-way cycle track on the northern side of the Seamus Ennis Road. A continuous footpath will run along the northern side of Seamus Ennis Road, positioned between Alpha Care and the Shamrock Lodge Bar.

The cycle track will be segregated from the carriageway by a 120-millimetre-high kerb, while the footpath will be separated from the cycle track by a 60-millimetre-high kerb. Existing standard and accessible parking bays on the northern side of Seamus Ennis Road specifically those in front of the Shamrock Lodge Bar and just east of the SuperValu car park entrance have been relocated to the Drogheda Mall car park.

The carriageway will continue to operate with a single traffic lane in each direction, as per the current layout. However, the existing right-turn pocket on Seamus Ennis Road, which currently provides access to the Drogheda Mall car park, will be reconfigured as part of the proposed design.

Just west of the entrance to SuperValu, the existing bus stop (Bus Stop ID 987) will be changed to an island bus stop featuring a four bay bus shelter with front supports and side panels. The shelter will be positioned on the pedestrian waiting area adjacent to the bus stop. In addition, a grass verge is proposed at the corner of the bus stop island to enhance the public realm. The bus stop island will have a width of 2.5 metres, with a 3-metre-wide footpath provided behind it. The 1.5-metre-wide cycle track will run to the rear of the island bus stop. A pedestrian crossing point will connect the footpath to the bus stop island, and red blister tactile paving will be installed at the crossing point.

As part of the proposed works, seven existing trees adjacent to the cycle track on the northern side of Seamus Ennis Road will be removed and five new trees will be provided within the grass verge in this area. At the western extent of Seamus Ennis Road, as shown on Drawing Number One, where the scheme commences, a landscaped grass verge is proposed. This will be complemented by the planting of two new trees on the northern side of Seamus Ennis Road.

The existing bus stop (Bus Stop ID 7845) located along the frontage of the Drogheda Mall car park will be changed to an island bus stop with a four bay bus shelter with front support posts and side panels. The shelter will be positioned within the pedestrian waiting area adjacent to the bus stop. In addition, a grass verge is proposed at the corner of the bus bay to enhance the overall streetscape. The island bus stop will have a width of 2.5 metres, with a 1.8-metre-wide footpath provided behind it. The cycle track, measuring 1.3 metres wide, will run to the rear of the island bus stop for a limited section. A pedestrian crossing point will connect the footpath to the island bus stop, and red blister tactile paving will be installed at the crossing point. At this location, the cycle track will be raised to the same level as both the footpath and the island bus stop for a short section.

At the junction of Seamus Ennis Road with Main Street, the carriageway will be raised for a length of 12 metres to match the level of the footpath. The raised crossing will be the same material as the existing carriageway. Buff blister tactile paving will be installed on each side of this raised crossing. Additionally, a small, landscaped grass verge has been introduced near the raised crossing, located immediately west of Main Street.

At the entrance to the Drogheda Mall car park, a continuous footpath will be provided. This continuous footpath will be complemented by granite setts installed in front of the vehicle stop line. Within the car park, the existing layout will be modified. The number of standard parking bays will be reduced from 52 to 38, while the number of accessible bays will increase from three to five, with two of these relocated from the northern side of Seamus Ennis Road. The design also includes the introduction of two new bicycle parking areas: one located near the main car park entrance beside Specsavers, providing capacity for 16 bicycles, and another positioned near the Main Street entrance, offering capacity for four bicycles. Additionally, the car park boundary adjacent to Drogheda Mall will be slightly reduced and upgraded, with a new grass verge proposed at the northern end to enhance the overall streetscape and improve visual quality.

## Drawing number two shows the junction of McKee Avenue, Jamestown Road (south and north) and Seamus Ennis Road, commonly known as the five-arm junction to Saint Girls Canice’s Girls National School.

Drawing number two illustrates the proposed changes at the five-arm junction formed by McKee Avenue, Jamestown Road (North and South), and Seamus Ennis Road.

Under the scheme, Jamestown Road (South), currently a two-lane northbound one-way road, will be redesigned as a mixed-traffic street where cyclists and vehicles share space. Vehicle through-traffic will no longer be permitted, and vehicle access will be limited to the car park entrances for local access. As the route progresses, the design introduces a short, dedicated two-way cycle-only section, highlighted with contrasting surfacing separated by a static bollard. Adjacent to the Bank of Ireland, a 6.7-metre wide pedestrian crossing will be installed across the cycle track, using the same material as the footpath and incorporating corduroy warning paving. The scheme includes landscaped areas with new green spaces, seating, two new trees along the grass verge, and a bicycle stand providing space for four bicycles. Additionally, four existing trees will be removed, and three new trees will be planted in front of the Bank of Ireland.

Moving east along Seamus Ennis Road, the cycle track continues with protective islands to ensure connectivity, while the existing traffic island across Jamestown Road will be removed and replaced with a 2.4-metre single-stage pedestrian crossing. At this point, the northbound cycle track splits into two dedicated lanes—one for cyclists travelling to McKee Avenue and another for right-turning cyclists towards Jamestown Road (North)—with widths varying from 1.5 to two metres to accommodate these movements.

Further north along McKee Avenue and Jamestown Road (North), single-stage pedestrian crossings measuring 2.4 metres in width and equipped with red blister tactile paving are proposed to improve accessibility. Existing pedestrian crossings across Seamus Ennis Road to McKee Avenue and across to Jamestown Road (North) on both sides of the junction will remain in place at their current width of 2.4 metres. To enhance safety and connectivity, dedicated two-metre-wide cycle lanes are proposed parallel to these pedestrian crossings on both sides of the junction.

In the existing condition, the carriageway and footpath are separated by a 120-millimetre kerb, while the cycle track on the southern side of Seamus Ennis Road continues with kerb protection. Under the proposed design, the cycle track will remain segregated from the carriageway by a 120-millimetre-high kerb, and the footpath will be separated from the cycle track by a 60-millimetre-high kerb.

At the north-western corner of the McKee Avenue junction, the pavement area in front of SuperValu will be redesigned. A new pedestrian ramp will be installed to provide safe access to the crossing points. Directly in front of the SuperValu entrance, a landscaped grass verge is proposed, featuring two newly planted trees. Adjacent to this verge, a bicycle parking area with capacity for ten bicycles will be provided, while the existing public artwork ‘’Finglas hands’’ will remain in place. At the north-eastern corner of the junction, located on Jamestown Road (North) where the clock tower stands, the proposal includes new landscaping, raised planters, seating, and the planting of one new tree within a grass verge. The clock tower will remain in its existing position.

Along McKee Avenue, five existing perpendicular parking spaces within the SuperValu car park will be reconfigured to provide two parallel wheelchair-accessible parking bays. One existing tree will be removed adjacent to the proposed parking bays and complementary planting will be provided in a more suitable location.

In front of Finglas Credit Union, the footpath will measure two metres in width. To ensure a clear and safe route for cyclists, three existing trees in this area will be removed, and new complementary planting will be provided in a more suitable location.  The four existing parking spaces on the northern side of the road will be retained, with a buffer separating them from the cycle track. The cycle track will remain segregated from the footpath by a 60-millimetre-high kerb.

Continuing east along the northern side of Seamus Ennis Road, the existing hard-landscaped area is proposed to be replaced with a grass verge. Two new trees are proposed as part of landscaping requirements.

On the southern side of Seamus Ennis Road, the proposed one-way westbound cycle track continues. The existing bus stop (Bus Stop ID 979) will be changed to a bus stop island, and the current shelter will be replaced with a new four bay bus shelter. The 1.8-metre wide cycle track will run behind the bus stop island and will rise to footpath level for a short section. A pedestrian crossing with red blister tactile paving will provide access between the footpath and the island bus stop. Four new trees will be planted along the grass verge in this area. Just west of the bus stop, two existing car parking spaces will be converted to an additional grass verge.

## Drawing number three shows Seamus Ennis Road between Saint Canice’s Girls National School and the junction with Clune Road.

Drawing number three illustrates the proposed changes along Seamus Ennis Road, extending eastward from Saint Canice’s Girls National School to the signalised junction with Clune Road. Along the northern side of Seamus Ennis Road, the one-way cycle track, which is 1.8 metres in width continues east, with a minimum footpath width of 1.9 metres being maintained throughout.

A 60-millimetre-high kerb will separate the footpath from the cycle track to ensure safe and clear delineation between pedestrian and cyclist movements.

The existing bus stop island (Bus Stop ID 988) and its associated shelter, currently located in front of Finglas Library, will be relocated approximately 25 metres to the east. The bus shelter will be upgraded to a four bay bus shelter as part of the works, and the proposed cycle track will run behind the new bus stop island. A pedestrian crossing point equipped with red blister tactile paving will connect the footpath to the bus stop. To accommodate the proposed bus stop arrangement, one existing tree adjacent to the proposed bus shelter will be removed, and three new trees will be planted within the grass verge as part of the landscape enhancement works. The existing accessible parking bay located immediately west of the library entrance will be relocated approximately 30 metres to the east to facilitate the revised layout. To accommodate this change, one existing tree will be removed.

On the southern side of Seamus Ennis Road, a one-way cycle track measuring 1.8 metres in width is proposed. This cycle track will be segregated from the carriageway by a 120-millimetre-high kerb. Between Clune Road junction and the proposed parallel pedestrian and cycle crossing located in front of Saint Canice’s National School, a grass verge will run adjacent to both sides of the cycle track, and all existing trees in this section will be retained. Immediately west of the school’s pedestrian entrance, a signalised pedestrian crossing and a parallel two-way cycle crossing are proposed adjacent to each other. Red blister tactile paving will be installed on either side of the pedestrian crossing. One existing tree on the northern side of Seamus Ennis Road will be removed to accommodate the new two-way cycle crossing.

The existing six parking spaces directly in front of the Saint Canice Girls National school entrance will be reconfigured into two standard parking bays and two wheelchair-accessible parking bays. The cycle track in this section will be separated from the footpath by a 60-millimetre-high kerb, and one additional tree will be removed and planted elsewhere as part of the scheme.

## Drawing number four shows the junction of Seamus Ennis Road, Clune Road and Glasanon Road.

Drawing number four illustrates the proposed changes at the junction of Seamus Ennis Road, Clune Road, and Glasanon Road, which forms the eastern limit of the scheme.

On the northern side of Seamus Ennis Road, the proposed cycle track continues and ramps down before the Clune Road junction, continues through the junction at road level, and terminates just before the residential access road on Ballygall Road West. All existing trees along this section will be retained, with two additional trees proposed within a green verge adjacent to the carriageway. The existing signalised pedestrian crossing arrangement around Clune Road junction will be retained.

On the southern side of the Seamus Ennis Road, the one-way cycle track continues through the Clune Road junction and will be separated from the carriageway by a 120-millimetre-high kerb and a green verge.

The existing bus stop (Bus Stop ID 7836) located on southern side of Seamus Ennis Road will be retained. As part of the proposed design, this bus stop will be converted into a bus stop island arrangement. The proposed one-way cycle track, measuring 1.5 metres in width, will run behind this bus stop, and a pedestrian crossing point with red tactile paving is proposed to facilitate safe access between the footpath and the bus stop. At this location, the existing grass verge with trees along the southern side of Seamus Ennis Road will be retained behind the cycle track.

## Drawing number five shows Jamestown Road (South) and the junction of Main Street.

Drawing number five illustrates the southern extent of the scheme. On Jamestown Road (South), the Finglas Village Centre and its car park are located, while the now-closed Drake Inn pub is situated on the western side. Vehicles exiting the Finglas Village Centre car park will be required to turn left on departure.

As part of the proposed works, Jamestown Road (South) will be converted from a one-way street to a two-way cul-de-sac, extending up to the Bank of Ireland. The existing bus stop and layover area along Jamestown Road (South) will be relocated to Seamus Ennis Road. Footpath and traffic lane widths will remain as they are, ensuring that existing access and movement along this section of Jamestown Road are fully maintained.

Continuous footpath entry treatments will be provided at the junctions of Main Street with Jamestown Road (South) and Church Street. In addition, the central traffic island on Main Street will be modified to facilitate right-turn movements from Jamestown Road (South) onto Main Street. The red tactile paving at the existing signalised pedestrian crossing on Main Street will also be enhanced.