# Vernon Avenue to Clontarf Road Active Travel Scheme.

# Written Description of General Arrangement Drawings and Visualisations.

Contents

Overview.	2

Drawing Number 1 Preliminary Design General Arrangement (Seafield Road Junction to Kincora Road Junction)	3

Drawing Number 2 Preliminary Design General Arrangement (Vernon Garden to Clontarf Road Junction).	4

Visualisation 1	6

Visualisation 2	7

Visualisation 3	7

Visualisation 4	7

Visualisation 5	8

## Overview.

The Vernon Avenue to Clontarf Road Active Travel Scheme runs along Vernon Avenue, starting 50 metres south of the Seafield Road Junction and continuing south towards Clontarf Road. The route connects residential streets, local shops, public transport, coastal amenities, and links directly with the Gracefield Road to Vernon Avenue Active Travel Scheme to the north and the East Coast Trail to the south.

The scheme provides walking and cycling facilities along a route of approximately 600 metres, connecting Seafield Road to Clontarf Road.

The drawing pack shows the full length of the scheme and includes changes at junctions, pedestrian crossings, bus stop areas, and sections of public space. The drawings show a new community space, some new and adjusted footpaths and pedestrian crossings along Vernon Avenue.

All references to “The Project” or “The Scheme” in this document describe the Vernon Avenue to Clontarf Road Active Travel Scheme only.

.

## Drawing Number 1 Preliminary Design General Arrangement (Seafield Road Junction to Kincora Road Junction)

General Overview
Drawing Number 1 starts at the Seafield Road / Vernon Avenue junction and continues south along Vernon Avenue, finishing 120 metres after the Kincora Road junction. The total length shown in this drawing is approximately 280 metres. The drawing is orientated with north to the left of the image.

Beginning at the left-hand side of the drawing, near the Seafield Road junction with Vernon Avenue, this drawing shows cycling provided within the carriageway. There is no protected cycle track on this section. Instead, cyclists share the road space with general traffic, supported by road markings and signage that indicate where cycling takes place within the carriageway. These markings continue consistently along the route to show that the street operates as a mixed traffic environment.

Existing footpaths along Vernon Avenue are retained where possible and adjusted where required. The section between Seafield Road and Kincora Road is narrow from property boundary to property boundary. The western footpath is shown to be 2.06 metres and the eastern footpath to be 1.74 metres.

Moving towards the right-hand side of the drawing, the section between Kincora Road junction with Vernon Avenue and Nolans of Clontarf is shown. There is no protected cycle track on this section either, with cyclists sharing the road space with general traffic. This is supported by road markings and signage that indicate where cycling takes place within the carriageway.

The section between Kincora Road and Nolans of Clontarf is slightly wider from property boundary to property boundary than the section between Seafield Road and Kincora Road. The western footpath is shown to be 3.32 metres and the eastern footpath to be 1.97 metres. The carriageway along this stretch is narrowed to five metres as a speed control measure to slow down vehicles.

There is a raised zebra crossing located approximately 11 metres to the north of the carpark entrance into Nolans of Clontarf. There is a ramp in the carriageway before and after this zebra crossing, and there is zebra line marking on this crossing extending from footpath to footpath. Red blister tactile paving is located in the footpath at each end of this raised zebra pedestrian. Buff coloured surfacing is shown on the drawing in the carriageway between Kincora Road junction and the raised zebra crossing as a further speed control measure to slow down vehicles.

Where existing lighting columns or signage would interfere with footpaths or pedestrian crossing points, they are relocated short distances. Any relocated columns and signs are placed outside the clear walking route, so that footpaths and pedestrian waiting areas remain unobstructed.

At the right-hand side of the drawing the start of a chicane is shown. This is incorporated to allow 12 parking spaces to remain along Vernon Avenue. This chicane starts at the southern end of Nolans of Clontarf and extends south towards St. Josephs Square.

## Drawing Number 2 Preliminary Design General Arrangement (Vernon Garden to Clontarf Road Junction).

General Overview.

Drawing Number 2 starts at the southern end of Nolans of Clontarf and continues south as far as Clontarf Road. The total length shown in this drawing is about 280 metres. The drawing is orientated with north to the left of the image.

Beginning at the left hand side of the drawing, the southern end of Nolans of Clontarf is shown. This is also the starting point for the chicane. This is incorporated in the carriageway to allow 12 parking spaces to remain along Vernon Avenue and extends south as far as St. Josephs Square. This acts as a give way arrangement, with cars only able to continue through when no oncoming cars are approaching.

Cycling is provided within the carriageway along this stretch of the drawing from Nolans of Clontarf at the left hand side, as far as Clontarf Park towards the right hand side of the drawing. There is no protected cycle track in this section. Instead, cyclists share the road space with general traffic, supported by road markings and signage that indicate where cycling takes place within the carriageway.

The western footpath is shown to be 2.39 metres wide and the eastern footpath to be 2.15 metres wide along this stretch.

At the right-hand side of the drawing, between Clontarf Park and Clontarf Road, a filtered permeability area is shown on the drawing. This means vehicles are not allowed to pass through this area, with only pedestrians and cyclists allowed in this zone. The existing footpath is retained along the building edge on both sides of the street within this zone to allow a continuous accessible route down to Clontarf Road.

Footpath buildouts are shown in front of the shops on both sides of the street in the filtered permeability area which creates space for benches, seating and outdoor dining. This is located at the footpath level with a kerb edge separating from the lower carriageway level. Between the footpath buildouts, there will be a space remaining in the middle 3.5 metres wide, allowing cyclists and emergency vehicles to continue through the space at the lower carriageway level.

* Pedestrians can step down from the footpath level onto the lower carriageway level within this filtered permeability area, but there will still be an accessible route at footpath level along the building edge on both sides of the street.

*Buff coloured asphalt is shown in the drawing within the filtered permeability area between Clontarf Park and Clontarf Road. This will be applied to the lower carriageway level and the new footpath buildouts in front of the shops along Vernon Avenue. The footpaths on either side of the street will remain grey concrete finish.

* At the southern end of Vernon Avenue in front of Kennedys Café. a flexible event space is shown in the drawing at the lower carriageway level. This will also have buff coloured asphalt. This space can be used for local events or markets and will have benches to provide resting places along the northern edge of the space. In front of the Sheds Pub, an outdoor dining area is shown at the lower carriageway level, with planters creating a boundary

*Dished kerbs with tactile paving are shown in the footpaths on both sides of the street at the northern end of the filtered permeability area, and further south where the building edge along Vernon Avenue ends, to allow pedestrians cross the street where cyclists will be passing through the space.

*Moving further to the right of the drawing at the southern end of Vernon Avenue where it meets Clontarf Road, a continuous footpath is shown which will be at the higher footpath level where cyclists will approach the cyclist crossing of Clontarf Road. This will be 3.5 metres in length which is shorter than typical for a continuous footpath and will have a ramp at each side to slow down cyclists approaching and passing through it. There will be tactile paving at both sides of the continuous footpath to indicate to pedestrians that they are passing an area where cyclists may be passing through. Pedestrians will have priority when passing through this continuous footpath, and cyclist will have to yield to pedestrians when crossing.

* A new pedestrian and cycle crossing is shown at the southern end of Vernon Avenue where it meets Clontarf Road, to facilitate crossing of Clontarf Road. This crossing includes tactile paving and signage and will connect Vernon Avenue with the Promenade and carpark to the south of Clontarf Road

* The existing East Coast trail which runs along the coast is shown in the drawing to be rerouted slightly, allowing space for the footpath to directly connect over to the carpark footpath along the Promenade.

* The drawing shows one accessible parking space approximately 15 metres to the west side of Vernon Avenue along Clontarf Road, with multiple loading bays alongside this accessible parking space. The existing accessible parking space in front of Kennedy’s café is proposed to be relocated in the south direction, and will be located along Clontarf Road, with a loading bay included alongside it. A new footpath with dropped kerb is proposed to the private vehicle entrance to the north east of The Sheds Pub just beside the loading bay.

*Two existing bus stops are retained along this section: Stop 1731 (route 130) is located on the north-west side of Clontarf Road. and Stop 1747 (route 130) is located on the south-west side of Clontarf Road, close to the pedestrian and cycle crossing at the Vernon Avenue junction. These stops remain on Clontarf Road and are accessed directly from the footpath. They are not island bus stops, and no changes are made to their basic layout.

## Visualisation 1 View looking north along Clontarf Road towards Vernon Avenue showing the proposed new community space

This image is a view of the completed scheme taken at eye level looking north west from Clontarf Road towards Vernon Avenue. New freestanding planters, benches and outdoor dining are shown to the community space. The accessible parking bay and loading bay to the east of Vernon Avenue along Clontarf Road is shown to the bottom of the image. The new pedestrian and cyclists crossing of Clontarf Road at the southern end of Vernon Avenue is shown to the left hand side of the image. People are shown using the space to the north of Clontarf Road, with children playing, people walking in and out of shops and some people sitting on benches.

## Visualisation 2 View looking south along Vernon Avenue showing start of ‘filtered permeability’ arrangement and proposed new community space

This image is a view showing the completed scheme taken at eye level of Vernon Avenue at the junction of Clontarf Park looking south towards Clontarf Promenade. It shows the start of the filtered permeability area, whereby only pedestrians and cyclists can pass through this zone going towards Clontarf Road to the south. Footpath buildouts are shown along the front of the shops with freestanding planters, benches and outdoor dining within these zones. Children are shown walking along the footpath to the right of the image. People are shown walking dogs, sitting on benches within the filtered permeability area. Cyclists are shown entering and existing the filtered permeability area between the footpath buildouts.

## Visualisation 3 View looking south along Vernon Avenue showing ‘mixed traffic’ street

This image is a view showing the completed scheme taken at eye level of Vernon Avenue from Nolan’s of Clontarf Supermarket looking south towards Clontarf Promenade. The image shows a stretch of Vernon Avenue which is a mixed traffic street, whereby cyclists share the carriageway with vehicles. The footpath continues uninterrupted to the left and right of the image along Vernon Avenue. The start of the chicane is shown where twelve parking spaces have been maintained along this street. There are small footpath buildouts shown which direct vehicles and cyclists past the parking spaces, with freestanding planters shown on top of these buildouts. Two cyclists are shown cycling along the street in the northbound and southbound directions. People are shown pushing buggies and walking dogs along the footpath to the right of the image.

## Visualisation 4 View looking south along Vernon Avenue showing ‘mixed traffic’ street outside ‘Nolan's of Clontarf’

This image is a view showing the completed scheme taken at eye level of Vernon Avenue at Kincora Road junction looking south towards Nolans of Clontarf and beyond towards Clontarf Promenade. The image shows a stretch of Vernon Avenue which is a mixed traffic street, whereby cyclists share the carriageway with vehicles. The footpath continues uninterrupted to the left and right of the image along Vernon Avenue. The carriageway is shown to be narrower along this stretch as a speed control measure. Buff coloured asphalt is shown as the surfacing on the carriageway. A raised zebra crossing is shown near the carpark entrance to Nolans of Clontarf. A car and two cyclists are shown in the carriageway, two people are shown running along the footpath to the right of the image, and a number of people are shown crossing the raised zebra crossing at the entrance to Nolans of Clontarf.

## Visualisation 5 Aerial view looking north from Clontarf Promenade towards Vernon Avenue showing the new community space

This image is an aerial view of the completed scheme looking north from Clontarf promenade towards Vernon Avenue. The image shows the proposed new filtered permeability area to the southern end of Vernon Avenue between Clontarf Park and Clontarf Road, where only pedestrians and cyclists can pass through this zone. The remainder of Vernon Avenue is shown extending to the north beyond this area. New freestanding planters, benches and outdoor dining are shown to the community space within the filtered permeability area. The existing footpaths are shown to continue along the building edge in front of the shops on both sides of the street along Vernon Avenue. The new cyclist and pedestrian crossing of Clontarf Road is shown towards the bottom of the image improving the connection of Vernon Avenue with Clontarf Promenade and the carpark to the south of Clontarf Road. The new continuous footpath crossing Vernon Avenue in the east west direction where it meets Clontarf Road is also shown. The accessible parking bays and loading bays are shown to the east and west of Vernon Avenue along Clontarf Road. People are shown using the space, with children playing, people walking in and out of shops and people sitting on benches.