TRL LIMITED

(C) COPYRIGHT 2006

CAPACITIES, QUEUES, AND DELAYS AT 3 OR 4-ARM MAJOR/MINOR PRIORITY JUNCTIONS

PICADY 5.1  ANALYSIS PROGRAM

RELEASE 4.0 (SEPT 2008)

ADAPTED FROM PICADY/3 WHICH IS CROWN COPYRIGHT

BY PERMISSION OF THE CONTROLLER OF HMSO

--------------------------------------------------------

FOR SALES AND DISTRIBUTION INFORMATION,

PROGRAM ADVICE AND MAINTENANCE CONTACT:

TRL SOFTWARE BUREAU

TEL: CROWTHORNE (01344) 770758, FAX: 770356

EMAIL: Software@trl.co.uk

--------------------------------------------------------

Run with file:-

"C:\Documents and Settings\Aiofe\Local Settings\temp\PICADY 5\PICADY5.vpi"

(drive-on-the-left) at 13:41:08 on Monday, 22 November 2010

.RUN INFORMATION

***************

RUN TITLE       : O'Denaney Gardens PM Peak 2032

LOCATION        : Infirmary Road/ Montpelllier Road

DATE            : 22/11/10

CLIENT          : Dublin City Council

ENUMERATOR      :

JOB NUMBER      : 110/A59

STATUS          : TIA

DESCRIPTION     :

.MAJOR/MINOR JUNCTION CAPACITY AND DELAY

***************************************

INPUT DATA

----------

MAJOR ROAD (ARM C) --------------------- MAJOR ROAD (ARM A)

I

I

I

I

I

I

MINOR ROAD (ARM B)

ARM A IS Infirmary Road North

ARM B IS Montpellier Road

ARM C IS Infirmary Road South

.STREAM LABELLING CONVENTION

---------------------------

STREAM  A-B  CONTAINS TRAFFIC GOING FROM ARM  A TO ARM B

STREAM  B-AC CONTAINS TRAFFIC GOING FROM ARM  B TO ARM A AND TO ARM  C

ETC.

.GEOMETRIC DATA

--------------

--------------------------------------------------------------------------------------

I                DATA ITEM                                       I   MINOR ROAD B    I

--------------------------------------------------------------------------------------

I  TOTAL MAJOR ROAD CARRIAGEWAY WIDTH                            I ( W  )  8.00 M.   I

I  CENTRAL RESERVE WIDTH                                         I (WCR )  0.00 M.   I

I                                                                I                   I

I  MAJOR ROAD RIGHT  TURN - WIDTH                                I (WC-B)  0.00 M.   I

I                         - VISIBILITY                           I (VC-B)120.00 M.   I

I                         - BLOCKS TRAFFIC                       I          YES      I

I                                                                I                   I

I  MINOR ROAD - VISIBILITY TO LEFT                               I (VB-C) 120.0 M.   I

I             - VISIBILITY TO RIGHT                              I (VB-A) 120.0 M.   I

I             - LANE 1 WIDTH                                     I (WB-C)  4.00 M.   I

I             - LANE 2 WIDTH                                     I (WB-A)  0.00 M.   I

--------------------------------------------------------------------------------------

.SLOPES AND INTERCEPT

--------------------

(NB:Streams may be combined, in which case capacity will be adjusted)

---------------------------------------------------------

I Intercept For Slope For Opposing   Slope For Opposing I

I STREAM B-C     STREAM  A-C          STREAM A-B        I

---------------------------------------------------------

I     769.51            0.27                0.11        I

---------------------------------------------------------

--------------------------------------------------------------------------------------------------

I Intercept For Slope For Opposing   Slope For Opposing   Slope For Opposing   Slope For OpposingI

I STREAM B-A     STREAM  A-C          STREAM A-B          STREAM  C-A          STREAM  C-B       I

--------------------------------------------------------------------------------------------------

I     635.97            0.27                0.11                  0.17                0.38       I

--------------------------------------------------------------------------------------------------

---------------------------------------------------------

I Intercept For Slope For Opposing   Slope For Opposing I

I STREAM C-B     STREAM  A-C          STREAM A-B        I

---------------------------------------------------------

I     643.46            0.23                0.23        I

---------------------------------------------------------

(NB These values do not allow for any site specific corrections)

.TRAFFIC DEMAND DATA

-------------------

-----------------------

I ARM I FLOW SCALE(%) I

-----------------------

I A   I      100      I

I B   I      100      I

I C   I      100      I

-----------------------

.Demand set:        O'Denaney Gardens

TIME PERIOD BEGINS 16.45 AND ENDS  18.15

LENGTH OF TIME PERIOD  -   90 MIN.

LENGTH OF TIME SEGMENT -   15 MIN.

.DEMAND FLOW PROFILES ARE SYNTHESISED FROM TURNING COUNT DATA

------------------------------------------------------------------------------------

I        I   NUMBER OF MINUTES FROM START WHEN    I   RATE OF FLOW (VEH/MIN)       I

I  ARM   I FLOW STARTS I TOP OF PEAK I FLOW STOPS I BEFORE I AT TOP  I AFTER       I

I        I   TO RISE   I  IS REACHED I FALLING    I  PEAK  I OF PEAK I PEAK        I

I        I             I             I            I        I         I             I

------------------------------------------------------------------------------------

I ARM  A I     15.00   I     45.00   I    75.00   I  8.66  I  12.99  I  8.66       I

I ARM  B I     15.00   I     45.00   I    75.00   I  2.89  I   4.33  I  2.89       I

I ARM  C I     15.00   I     45.00   I    75.00   I 10.82  I  16.24  I 10.82       I

------------------------------------------------------------------------------------

.Demand set:        O'Denaney Gardens

-----------------------------------------------------------

I                    I         TURNING PROPORTIONS        I

I                    I         TURNING COUNTS             I

I                    I        (PERCENTAGE OF H.V.S)       I

I                    --------------------------------------

I        TIME        I FROM/TO I ARM  A I ARM  B I ARM  C I

-----------------------------------------------------------

I   16.45 - 17.00    I         I        I        I        I

I                    I ARM  A  I  0.000 I  0.035 I  0.965 I

I                    I         I    0.0 I   24.0 I  669.0 I

I                    I         I (  0.0)I ( 10.0)I ( 10.0)I

I                    I         I        I        I        I

I                    I ARM  B  I  0.095 I  0.000 I  0.905 I

I                    I         I   22.0 I    0.0 I  209.0 I

I                    I         I ( 10.0)I (  0.0)I ( 10.0)I

I                    I         I        I        I        I

I                    I ARM  C  I  0.759 I  0.241 I  0.000 I

I                    I         I  657.0 I  209.0 I    0.0 I

I                    I         I ( 10.0)I ( 10.0)I (  0.0)I

I                    I         I        I        I        I

-----------------------------------------------------------

.

QUEUE AND DELA INFORMATION FOR EACH 15 MIN TIME SEGMENT

--------------------------------------------------------

FOR COMBINED DEMAND SETS

AND FOR TIME PERIOD        1

.------------------------------------------------------------------------------------------------------------------

I  TIME      DEMAND  CAPACITY  DEMAND/   PEDESTRIAN  START   END       DELAY     GEOMETRIC DELAY   AVERAGE DELAY I

I         (VEH/MIN) (VEH/MIN) CAPACITY      FLOW     QUEUE  QUEUE    (VEH.MIN/      (VEH.MIN/      PER ARRIVING  I

I                               (RFC)    (PEDS/MIN) (VEHS) (VEHS)  TIME SEGMENT)  TIME SEGMENT)    VEHICLE (MIN) I

I 16.45-17.00                                                                                                    I

I   B-AC      2.90      8.62    0.336                0.00   0.50        7.1                            0.17      I

I   C-A       8.24                                                                                               I

I   C-B       2.62      7.77    0.338                0.00   0.50        7.1                            0.19      I

I   A-B       0.30                                                                                               I

I   A-C       8.39                                                                                               I

I                                                                                                                I

.------------------------------------------------------------------------------------------------------------------

.------------------------------------------------------------------------------------------------------------------

I  TIME      DEMAND  CAPACITY  DEMAND/   PEDESTRIAN  START   END       DELAY     GEOMETRIC DELAY   AVERAGE DELAY I

I         (VEH/MIN) (VEH/MIN) CAPACITY      FLOW     QUEUE  QUEUE    (VEH.MIN/      (VEH.MIN/      PER ARRIVING  I

I                               (RFC)    (PEDS/MIN) (VEHS) (VEHS)  TIME SEGMENT)  TIME SEGMENT)    VEHICLE (MIN) I

I 17.00-17.15                                                                                                    I

I   B-AC      3.46      7.98    0.433                0.50   0.75       10.7                            0.22      I

I   C-A       9.84                                                                                               I

I   C-B       3.13      7.39    0.424                0.50   0.72       10.3                            0.23      I

I   A-B       0.36                                                                                               I

I   A-C      10.02                                                                                               I

I                                                                                                                I

.------------------------------------------------------------------------------------------------------------------

.------------------------------------------------------------------------------------------------------------------

I  TIME      DEMAND  CAPACITY  DEMAND/   PEDESTRIAN  START   END       DELAY     GEOMETRIC DELAY   AVERAGE DELAY I

I         (VEH/MIN) (VEH/MIN) CAPACITY      FLOW     QUEUE  QUEUE    (VEH.MIN/      (VEH.MIN/      PER ARRIVING  I

I                               (RFC)    (PEDS/MIN) (VEHS) (VEHS)  TIME SEGMENT)  TIME SEGMENT)    VEHICLE (MIN) I

I 17.15-17.30                                                                                                    I

I   B-AC      4.24      6.97    0.608                0.75   1.47       20.1                            0.35      I

I   C-A      12.06                                                                                               I

I   C-B       3.84      6.85    0.559                0.72   1.21       16.9                            0.32      I

I   A-B       0.44                                                                                               I

I   A-C      12.28                                                                                               I

I                                                                                                                I

.------------------------------------------------------------------------------------------------------------------

.------------------------------------------------------------------------------------------------------------------

I  TIME      DEMAND  CAPACITY  DEMAND/   PEDESTRIAN  START   END       DELAY     GEOMETRIC DELAY   AVERAGE DELAY I

I         (VEH/MIN) (VEH/MIN) CAPACITY      FLOW     QUEUE  QUEUE    (VEH.MIN/      (VEH.MIN/      PER ARRIVING  I

I                               (RFC)    (PEDS/MIN) (VEHS) (VEHS)  TIME SEGMENT)  TIME SEGMENT)    VEHICLE (MIN) I

I 17.30-17.45                                                                                                    I

I   B-AC      4.24      6.96    0.609                1.47   1.51       22.4                            0.37      I

I   C-A      12.06                                                                                               I

I   C-B       3.84      6.85    0.559                1.21   1.24       18.4                            0.33      I

I   A-B       0.44                                                                                               I

I   A-C      12.28                                                                                               I

I                                                                                                                I

.------------------------------------------------------------------------------------------------------------------

.------------------------------------------------------------------------------------------------------------------

I  TIME      DEMAND  CAPACITY  DEMAND/   PEDESTRIAN  START   END       DELAY     GEOMETRIC DELAY   AVERAGE DELAY I

I         (VEH/MIN) (VEH/MIN) CAPACITY      FLOW     QUEUE  QUEUE    (VEH.MIN/      (VEH.MIN/      PER ARRIVING  I

I                               (RFC)    (PEDS/MIN) (VEHS) (VEHS)  TIME SEGMENT)  TIME SEGMENT)    VEHICLE (MIN) I

I 17.45-18.00                                                                                                    I

I   B-AC      3.46      7.98    0.434                1.51   0.79       12.5                            0.23      I

I   C-A       9.84                                                                                               I

I   C-B       3.13      7.39    0.424                1.24   0.76       12.0                            0.24      I

I   A-B       0.36                                                                                               I

I   A-C      10.02                                                                                               I

I                                                                                                                I

.------------------------------------------------------------------------------------------------------------------

.------------------------------------------------------------------------------------------------------------------

I  TIME      DEMAND  CAPACITY  DEMAND/   PEDESTRIAN  START   END       DELAY     GEOMETRIC DELAY   AVERAGE DELAY I

I         (VEH/MIN) (VEH/MIN) CAPACITY      FLOW     QUEUE  QUEUE    (VEH.MIN/      (VEH.MIN/      PER ARRIVING  I

I                               (RFC)    (PEDS/MIN) (VEHS) (VEHS)  TIME SEGMENT)  TIME SEGMENT)    VEHICLE (MIN) I

I 18.00-18.15                                                                                                    I

I   B-AC      2.90      8.61    0.336                0.79   0.52        8.1                            0.18      I

I   C-A       8.24                                                                                               I

I   C-B       2.62      7.77    0.338                0.76   0.52        8.1                            0.20      I

I   A-B       0.30                                                                                               I

I   A-C       8.39                                                                                               I

I                                                                                                               I

.------------------------------------------------------------------------------------------------------------------

QUEUE FOR STREAM    B-AC

-------------------------

TIME          NO. OF

SEGMENT       VEHICLES

ENDING        IN QUEUE

17.00           0.5

17.15           0.7    *

17.30           1.5    *

17.45           1.5    **

18.00           0.8    *

18.15           0.5    *

.

QUEUE FOR STREAM    C-B

-------------------------

TIME          NO. OF

SEGMENT       VEHICLES

ENDING        IN QUEUE

17.00           0.5

17.15           0.7    *

17.30           1.2    *

17.45           1.2    *

18.00           0.8    *

18.15           0.5    *

.

QUEUEING DELAY INFORMATION OVER WHOLE PERIOD

--------------------------------------------

---------------------------------------------------------------------------

I STREAM I   TOTAL DEMAND  I   * QUEUEING *      I * INCLUSIVE QUEUEING * I

I        I                 I    * DELAY *        I       * DELAY *        I

I        I----------------------------------------------------------------I

I        I  (VEH)  (VEH/H) I  (MIN)    (MIN/VEH) I    (MIN)     (MIN/VEH) I

---------------------------------------------------------------------------

I  B-AC  I  318.0 I  212.0 I    80.9 I    0.25   I      80.9  I    0.25   I

I  C-A   I  904.3 I  602.9 I         I           I            I           I

I  C-B   I  287.7 I  191.8 I    72.8 I    0.25   I      72.9  I    0.25   I

I  A-B   I   33.0 I   22.0 I         I           I            I           I

I  A-C   I  920.8 I  613.9 I         I           I            I           I

---------------------------------------------------------------------------

I  ALL   I 2463.8 I 1642.5 I   153.7 I    0.06   I     153.7  I    0.06   I

---------------------------------------------------------------------------

* DELAY IS THAT OCCURRING ONLY WITHIN THE TIME PERIOD

* INCLUSIVE DELAY INCLUDES DELAY SUFFERED BY VEHICLES

WHICH ARE STILL QUEUEING AFTER THE END OF THE TIME PERIOD

* THESE WILL ONLY BE SIGNIFICANTLY DIFFERENT IF THERE IS

A LARGE QUEUE REMAINING AT THE END OF THE TIME PERIOD.

*******END OF RUN*******

============================================= end of file ===============================================